M2 Fuse Lighter

Items Needed

- Flange Bearing 3/8 x 1/2 x 5/8 x1 (typically in the nut and bolt section)

- Clevis Pin 1/4x2

- Cotter Pin 1/16x3/4

- Tin or Aluminum 2.5”x2” (I used .008mm tick tin sheet) Thinner = easier

- Key ring  5/8” or 3/4" (used mine from a set of spare padlock keys)

- Dowel rod 3/8”  1” long

- Electrical tape

- Optional Dowel rod 1/2"  3-4” long (Used to help roll tin)

- Insert dowel half way on the flanged side of the bearing. Will be a snug fit.

If needed compress dowel slightly in a vice or sand down

- Roll tin or aluminum around 1/2" dowel rod to make a 2.5” tube.

To make a better seal remove dowel after initial roll and roll slightly tighter manually.

After rolling you can apply super glue to joint to help seal. (I didn’t)

- Wrap electric tape around dowel on Flange until tin roll fits snugly. 9 times around worked for me. No need to trim any tape hanging past edge of dowel rod, this extra tape will help hold the clevis pin later.

- Wrap electric tape around clevis pin at the bottom of the first pin hole until pin fits snugly in rolled tube. Top of tape will be level with edge of tube.

- Trim Cotter pin to around 5/8” or width of tube. Place on ring.

- Paint OD and insert pin.

- To make paper tube. Cut 1.25”x3” strip of card stock, roll and glue into a tube using the bearing for sizing.